Lighting power formulation

The control vector specified discrete output levels for three ceiling-light zones and 24 task lights. Task lights at unoccupied workstations remained off. Each task-light level added 1.5 W, and each ceiling-zone level added 13.2 W. Total lighting power was calculated as:

|  | P = 13.2 ∑j=13Lc,j + 1.5 ∑i=124Lt,i. | (1) |
| --- | --- | --- |

Here, P is the total lighting power in watts; Lc,j is the discrete output level of ceiling-light zone j; and Lt,i is the discrete output level of task light i. The indices j = 1, ..., 3 and i = 1, ..., 24 represent the three ceiling-light zones and 24 workstations, respectively. For an unoccupied workstation, Lt,i = 0.

AUTHOR INPUT: Add the allowable discrete ranges for the ceiling-light and task-light levels.

Penalty-based objective formulation

The GA minimized a scalar fitness function combining total lighting power with quadratic penalties for unmet illuminance and uniformity requirements. For each performance metric, the normalized shortfall was calculated as:

|  | vk = max(0, rk − xkrk). | (2) |
| --- | --- | --- |

where xk is the simulated value of metric k, rk is its required threshold, and vk is the normalized shortfall. A requirement that was satisfied produced vk = 0; otherwise, the shortfall increased with the relative deficit.

The required thresholds were 750 lx for E1, 500 lx for E0, 0.70 for U1, and 0.50 for U2. Using a quadratic penalty exponent and a power-dependent scaling factor s = max(1, P), the fitness was:

|  | F = P + s[200vE12 + 150vE02 + 200vU12 + 150vU22]. | (3) |
| --- | --- | --- |

Here, F is the scalar fitness minimized by Galapagos, P is the total lighting power, and s scales the penalty weights relative to the power magnitude. A configuration was feasible when all four requirements were satisfied. For feasible configurations, all penalty terms were zero and the fitness equalled total lighting power. Infeasible configurations received quadratic penalties that increased with the magnitude of each normalized violation.

AUTHOR CHECK: The supplied code uses illuminance thresholds of 750 and 500 lx, although its comments state 500 and 300 lx. It also contains illu0 = num(illu2); verify whether this should be illu0 = num(illu0), and confirm which version generated the reported results.